BLE Debugging Note

Project Files

All of the necessary project files are contained in this patch: File:BLE Bridge.zip

To apply the patch, open the .zip file and copy the Project folder to C:\Texas Instruments\BLE-CC254x-1.4.0 choosing to replace all conflicting files.

Once the project is opened with IAR, you can select the appropriate serial protocol to use by selecting the CC254-SPI or CC2541-UART configurations.

If you are using IAR 8.30, you will need to modify your .xcl files to remove the _NR_OF_VIRTUAL_REGISTERS symbol, as described here: e2e.ti.com/support/low_power_rf/f/538/t/303921.aspx.

GAP_DeviceInit

shcho@shcho-PC-DELL /c/Work.git/ble/BLE-CC254x-1.4.0

$ grep -rn --exclude=*.map --exclude=*.pb* --exclude=*.[rd]51 "GAP_DeviceInit" ./Projects/

./Projects/ble/HostTestApp/Source/hci_ext_app.c:1386:        stat = GAP_DeviceInit( hciExtApp_TaskID, profileRole, pBuf[1], IRK, SRK, &hciExtSignCounter );

./Projects/ble/Profiles/Roles/broadcaster.c:631:  VOID GAP_DeviceInit( gapRole_TaskID,

./Projects/ble/Profiles/Roles/central.c:152:  return GAP_DeviceInit( gapCentralRoleTaskId, GAP_PROFILE_CENTRAL,

./Projects/ble/Profiles/Roles/gap.c:81:bStatus_t GAP_DeviceInit(  uint8 taskID,

./Projects/ble/Profiles/Roles/observer.c:117:  return GAP_DeviceInit( gapObserverRoleTaskId, GAP_PROFILE_OBSERVER,

./Projects/ble/Profiles/Roles/peripheral.c:1194:  VOID GAP_DeviceInit( gapRole_TaskID,

./Projects/ble/Profiles/Roles/peripheralBroadcaster.c:1059:  VOID GAP_DeviceInit( gapRole_TaskID,

pTaskEventHandlerFn

85 /*********************************************************************

86  * GLOBAL VARIABLES

87  */

88

89 // The order in this table must be identical to the task initialization calls below in osalInitTask.

90 const pTaskEventHandlerFn tasksArr[] =

91 {

92   LL_ProcessEvent,                                                  // task 0

93   Hal_ProcessEvent,                                                 // task 1

94   HCI_ProcessEvent,                                                 // task 2

95 #if defined ( OSAL_CBTIMER_NUM_TASKS )

96   OSAL_CBTIMER_PROCESS_EVENT( osal_CbTimerProcessEvent ),           // task 3

97 #endif

98   L2CAP_ProcessEvent,                                               // task 4

99   GAP_ProcessEvent,                                                 // task 5

100   GATT_ProcessEvent,                                                // task 6

101   SM_ProcessEvent,                                                  // task 7

102   GAPRole_ProcessEvent,                                             // task 8

103   GAPBondMgr_ProcessEvent,                                          // task 9

104   GATTServApp_ProcessEvent,                                         // task 10

105   SerialInterface_ProcessEvent,                                     // task 11

106   BLE_Bridge_ProcessEvent                                           // task 12

107 };

108

109 const uint8 tasksCnt = sizeof( tasksArr ) / sizeof( tasksArr[0] );

110 uint16 *tasksEvents;

111

112 /*********************************************************************

113  * FUNCTIONS

./BLE_Bridge/Source/OSAL_BLE_Bridge.c

78

79 /*********************************************************************

80  * GLOBAL VARIABLES

81  */

82

83 // The order in this table must be identical to the task initialization calls below in osalInitTask.

84 const pTaskEventHandlerFn tasksArr[] =

85 {

86   LL_ProcessEvent,

87   Hal_ProcessEvent,

88   HCI_ProcessEvent,

89 #if defined ( OSAL_CBTIMER_NUM_TASKS )

90   OSAL_CBTIMER_PROCESS_EVENT( osal_CbTimerProcessEvent ),

91 #endif

92   L2CAP_ProcessEvent,

93   GAP_ProcessEvent,

94   GATT_ProcessEvent,

95   SM_ProcessEvent,

96   GAPRole_ProcessEvent,

97   GAPBondMgr_ProcessEvent,

98   GATTServApp_ProcessEvent,

99   HeartRate_ProcessEvent

100 };

101

102 const uint8 tasksCnt = sizeof( tasksArr ) / sizeof( tasksArr[0] );

103 uint16 *tasksEvents;

104

105 /*********************************************************************

106  * FUNCTIONS

./HeartRate/Source/OSAL_heartrate.c